PADP (18CS73)

Computer Architecture: A Quantitative Approach, Fifth Edition

UNIT 2 – Multiprocessors and Thread-Level Parallelism (Chapter 5)

Structured Exam-Revision Notes

Based on slides by Dr. Minal Moharir (Hennessy & Patterson, Ch. 5)

## Contents

- Part 1 – Introduction: Thread-Level Parallelism & Types of Multiprocessors

- Part 2 – Cache Coherence & Consistency (Concepts)

- Part 3 – Snoopy Coherence Protocols (MSI, MESI, Extensions)

- Part 4 – Performance of Symmetric Shared-Memory Multiprocessors

- Part 5 – Distributed Shared Memory & Directory-Based Coherence

- Part 6 – Synchronization (Locks, Atomic Operations, LL/SC)

- Part 7 – Models of Memory Consistency (SC, TSO, XC, Relaxed)

- Quick Revision – Comparison Tables & Key Terms

# PART 1 – Introduction: Thread-Level Parallelism

## 1.1 Thread-Level Parallelism (TLP)

- Thread-Level parallelism has multiple program counters and uses the MIMD model.

- Targeted for tightly-coupled shared-memory multiprocessors.

- For n processors, need n threads.

DEFINITION Grain size – the amount of computation assigned to each thread.

- Threads can be used for data-level parallelism, but the overheads may outweigh the benefit.

## 1.2 Types of Shared-Memory Multiprocessors

|  | Symmetric Multiprocessors (SMP) | Distributed Shared Memory (DSM) |
| --- | --- | --- |
| Cores | Small number of cores | Larger scale |
| Memory | Share single memory with uniform memory latency | Memory distributed among processors |
| Latency | Uniform (also called UMA) | Non-uniform memory access/latency (NUMA) |
| Connection | Shared bus / shared cache; private caches per processor (one or more levels) | Processors connected via direct (switched) and non-direct (multi-hop) interconnection networks |

- SMP structure: processors, each with one or more levels of private cache, a shared cache, main memory and I/O system.

- DSM structure: each node = multicore MP + memory + I/O; nodes joined by an interconnection network.

## 1.3 Role of Caches

- Caches serve to: increase bandwidth versus bus/memory; reduce latency of access; valuable for both private data and shared data.

- Question raised: what about cache coherence and consistency?

SECTION SUMMARY TLP = MIMD with multiple PCs, needs n threads for n processors. SMP = small, single memory, uniform latency; DSM = distributed memory, NUMA, interconnection network. Caches help but create the coherence/consistency problem.

# PART 2 – Cache Coherence & Consistency (Concepts)

## 2.1 The Cache Coherence Problem

- Processors may see different values through their caches:

| Time | Event | Cache contents for processor A | Cache contents for processor B | Memory contents for location X |
| --- | --- | --- | --- | --- |
| 0 |  |  |  | 1 |
| 1 | Processor A reads X | 1 |  | 1 |
| 2 | Processor B reads X | 1 | 1 | 1 |
| 3 | Processor A stores 0 into X | 0 | 1 (stale!) | 0 |

KEY POINT After time 3, processor B still holds the old value 1 in its cache, while memory and A hold 0 – B sees a stale value.

## 2.2 Coherence vs Consistency

| Coherence | Consistency |
| --- | --- |
| All reads by any processor must return the most recently written value. Writes to the same location by any two processors are seen in the same order by all processors. | Determines when a written value will be returned by a read. If a processor writes location A followed by location B, any processor that sees the new value of B must also see the new value of A. |

## 2.3 What does Coherence mean?

- Informally: “Any read must return the most recent write” – too strict and too difficult to implement.

- Better: “Any write must eventually be seen by a read”; all writes are seen in proper order (“serialization”).

### Two rules to ensure this

- “If P writes x and P1 reads it, P’s write will be seen by P1 if the read and write are sufficiently far apart.”

- Writes to a single location are serialized: seen in one order – the latest write will be seen. Otherwise one could see writes in illogical order (could see an older value after a newer value).

### Further points on coherence vs consistency

- The goal of cache coherence is to make caches in multi-core systems invisible.

- Coherence deals with one block at a time and is silent on the interaction of accesses to multiple cache blocks.

- It is possible to implement a memory system without coherence and even without caches.

- Cache coherence is not equal to memory consistency – use coherence as a black box that implements Single-Writer-Multiple-Readers invariants.

## 2.4 Enforcing Coherence

- Coherent caches provide:

  - Migration – movement of data

  - Replication – multiple copies of data

### Cache coherence protocols

| Protocol | Idea | Notes |
| --- | --- | --- |
| Directory based | Sharing status of each block kept in one location (1 centralized place, logically) | Distributed memory ⇒ distributed directory for scalability (avoids bottlenecks). Sends point-to-point requests to processors via network. Scales better than snooping. Actually existed before snooping-based schemes. |
| Snooping | Each core tracks sharing status of each block | Send all requests for data to all processors; processors snoop to see if they have a copy and respond accordingly. Requires broadcast, since caching information is at processors. Works well with bus (natural broadcast medium). Dominates for small-scale machines (most of the market). |

SECTION SUMMARY Coherence = reads return latest write + writes to a location serialized. Consistency = when/in what order writes become visible across locations. Two hardware solutions: snooping (broadcast, small machines) and directory (point-to-point, scalable).

# PART 3 – Snoopy Coherence Protocols

## 3.1 Write Invalidate vs Write Update

|  | Write Invalidate | Write Update |
| --- | --- | --- |
| On a write | Invalidate all other copies | Update all copies |
| Serialization | Use bus itself to serialize – the write cannot complete until bus access is obtained | — |
| Result (P1 writes X1) | P1 holds X1; caches of P2, P3 become I (invalid); memory holds X1 | All caches (P1, P2, P3) hold X1 |

### Write-invalidate example

| Processor activity | Bus activity | Contents of processor A’s cache | Contents of processor B’s cache | Contents of memory location X |
| --- | --- | --- | --- | --- |
|  |  |  |  | 0 |
| Processor A reads X | Cache miss for X | 0 |  | 0 |
| Processor B reads X | Cache miss for X | 0 | 0 | 0 |
| Processor A writes a 1 to X | Invalidation for X | 1 | (invalidated) | 0 |
| Processor B reads X | Cache miss for X | 1 | 1 | 1 |

### Locating an item when a read miss occurs

- In a write-back cache, the updated value must be sent to the requesting processor.

- Cache lines are marked as shared or exclusive/modified.

- Only writes to shared lines need an invalidate broadcast; after this, the line is marked as exclusive.

- Hardware: each private cache has an n-bit coherence state per block; a bus snoop watches cache transactions. A controller updates the state of each cache block in response to processor and snoop events and updates the bus transaction (state-transition mechanism for each cache block).

## 3.2 The 3-State (MSI) Write-Invalidate Protocol

| State | Meaning |
| --- | --- |
| Invalid (I) | Block not present / not valid in this cache |
| Shared (S, read only) | Block may be in several caches; read only |
| Exclusive / Modified (M, read/write) | Only this cache has the block; read/write (memory out of date) |

### State transitions based on requests from the CPU

| Current state | CPU event | Action | Next state |
| --- | --- | --- | --- |
| Invalid | CPU read | Place read miss on bus | Shared |
| Invalid | CPU write | Place write miss on bus | Exclusive |
| Shared | CPU read hit | Read data in local cache | Shared |
| Shared | CPU read miss | Place read miss on bus | Shared |
| Shared | CPU write (hit) | Place invalidate on bus | Exclusive |
| Shared | CPU write miss | Place write miss on bus | Exclusive |
| Exclusive | CPU read hit / CPU write hit | Use local cache | Exclusive |
| Exclusive | CPU read miss | Write-back block; place read miss on bus | Shared |
| Exclusive | CPU write miss | Write-back cache block; place write miss on bus | Exclusive |

- [The slide series builds this diagram step by step: first the read transitions (slide 19), then adds writes (slides 20–21). The table above merges them.]

### Cache behaviour summary table (requests from processor and bus)

| Request | Source | State of addressed cache block | Type of cache action | Function and explanation |
| --- | --- | --- | --- | --- |
| Read hit | Processor | Shared or modified | Normal hit | Read data in local cache. |
| Read miss | Processor | Invalid | Normal miss | Place read miss on bus. |
| Read miss | Processor | Shared | Replacement | Address conflict miss: place read miss on bus. |
| Read miss | Processor | Modified | Replacement | Address conflict miss: write-back block, then place read miss on bus. |
| Write hit | Processor | Modified | Normal hit | Write data in local cache. |
| Write hit | Processor | Shared | Coherence | Place invalidate on bus. These operations are often called upgrade or ownership misses, since they do not fetch the data but only change the state. |
| Write miss | Processor | Invalid | Normal miss | Place write miss on bus. |
| Write miss | Processor | Shared | Replacement | Address conflict miss: place write miss on bus. |
| Write miss | Processor | Modified | Replacement | Address conflict miss: write-back block, then place write miss on bus. |
| Read miss | Bus | Shared | No action | Allow shared cache or memory to service read miss. |
| Read miss | Bus | Modified | Coherence | Attempt to share data: place cache block on bus and change state to shared. |
| Invalidate | Bus | Shared | Coherence | Attempt to write shared block; invalidate the block. |
| Write miss | Bus | Shared | Coherence | Attempt to write shared block; invalidate the cache block. |
| Write miss | Bus | Modified | Coherence | Attempt to write block that is exclusive elsewhere; write-back the cache block and make its state invalid in the local cache. |

## 3.3 Complications and Extensions

### Complications for the basic MSI protocol

- Operations are not atomic – e.g. detect miss, acquire bus, receive a response.

  - Creates possibility of deadlock and races.

  - One solution: processor that sends invalidate can hold bus until other processors receive the invalidate.

### Extensions

- MESI protocol: add Exclusive state to indicate a clean block in only one cache – prevents needing to write invalidate on a write.

- Owned state (MOESI).

### Scaling snooping – coherence protocol extensions

- Shared memory bus and snooping bandwidth is the bottleneck for scaling symmetric multiprocessors. Remedies:

  - Duplicating tags

  - Place directory in outermost cache

  - Use crossbars or point-to-point networks with banked memory (processors with private caches → interconnection network → shared cache banks 0–3 → memory and I/O)

### AMD Opteron (example)

- Memory directly connected to each multicore chip in a NUMA-like organization.

- Implements the coherence protocol using point-to-point links.

- Uses explicit acknowledgements to order operations.

MNEMONIC MSI = Modified, Shared, Invalid. MESI adds Exclusive (clean, single copy). MOESI adds Owned.

SECTION SUMMARY Snooping: all caches watch the bus. Write invalidate is the common approach (write update = send new value to all). MSI has 3 states per block; writes to Shared blocks need an invalidate. Bus bandwidth limits scaling – use duplicated tags, outer-cache directory, crossbars/point-to-point.

# PART 4 – Performance of Symmetric Shared-Memory Multiprocessors

- Coherence influences cache miss rate.

## 4.1 Coherence Misses

| Type | When it occurs |
| --- | --- |
| True sharing misses | Write to shared block (transmission of invalidation) Read an invalidated block |
| False sharing misses | Read an unmodified word in an invalidated block (the block was invalidated because a different word in it was written) |

KEY POINT True sharing = the same word is really shared. False sharing = different words that merely sit in the same block.

## 4.2 Performance Study: Commercial Workload

The slides show four charts. [Observations read from the charts:]

| Chart | Axis / variable | Observation |
| --- | --- | --- |
| Normalized execution time (slide 27) | L3 cache size: 1, 2, 4, 8 MB | Time falls as L3 grows (mainly the memory-access component shrinks); instruction execution and L2/L3 access stay roughly constant. Components: idle, PAL code, memory access, L2/L3 cache access, instruction execution. |
| Memory cycles per instruction (slide 28) | Cache size: 1, 2, 4, 8 MB | Total falls with bigger cache (instruction, capacity/conflict, compulsory misses drop) but true-sharing component stays about the same. |
| Memory cycles per instruction (slide 29) | Processor count: 1, 2, 4, 6, 8 | Memory cycles per instruction rise with processor count, mainly because true-sharing misses grow. |
| Misses per 1000 instructions (slide 30) | Block size: 32, 64, 128, 256 bytes | Misses per 1000 instructions fall as block size increases; true-sharing misses decrease with larger blocks. |

- Miss categories in the charts: Instruction, Capacity/conflict, Compulsory, False sharing, True sharing.

SECTION SUMMARY Coherence adds true- and false-sharing misses. A bigger cache does not remove true-sharing misses; more processors increase them.

# PART 5 – Distributed Shared Memory & Directory-Based Coherence

## 5.1 Directory Protocols – Basics

- Directory keeps track of every block: which caches have each block; dirty status of each block.

- Implement in shared L3 cache: keep a bit vector of size = number of cores for each block in L3 – not scalable beyond shared L3.

- Implement in a distributed fashion: each node (multicore processor + caches + memory + I/O) has its own directory, all connected by an interconnection network.

- Maintain the state information explicitly: associated with each memory block; records the state of each block in cache. Directory entry = presence bits p[1..n] + dirty bit.

- On a cache miss, the node communicates with the directory: determines where the valid cache copies are (if any); determines what further actions need to be taken; performs a set of transactions to keep the directory up-to-date.

| Term | Meaning |
| --- | --- |
| Home node | Node in whose memory chunk the block is allocated |
| Owner node | Node that currently holds a valid copy of the block |
| CC | Cache controller (one per node) |

## 5.2 Directory Block States

| State | Meaning |
| --- | --- |
| Shared | One or more nodes have the block cached, value in memory is up-to-date. Directory stores the set of node IDs. |
| Uncached | No node has a copy of the block |
| Modified | Exactly one node has a copy of the cache block, value in memory is out-of-date. Directory stores the owner node ID. |

- The directory maintains block states and sends invalidation messages.

## 5.3 Cache-Side 3-State Protocol (write-back, invalidation-based)

| State | Event | Action | Next state |
| --- | --- | --- | --- |
| Invalid | CPU read miss | Send read miss message | Shared |
| Invalid | CPU write miss | Send write miss message | Modify |
| Shared | CPU read hit | — | Shared |
| Shared | CPU read miss | Send read miss message | Shared |
| Shared | CPU write hit | Send invalidate message | Modify |
| Shared | CPU write miss | Send write miss message | Modify |
| Shared | Invalidate (from directory) | — | Invalid |
| Modify | CPU read hit / write hit | — | Modify |
| Modify | CPU read miss | WB block; send read miss | Shared |
| Modify | CPU write miss | WB block; send write miss | Modify |
| Modify | Fetch (from directory) | WB block | Shared |
| Modify | Fetch/invalidate (from directory) | WB block | Invalid |

## 5.4 Messages

| Message type | Source | Destination | Message contents | Function of this message |
| --- | --- | --- | --- | --- |
| Read miss | Local cache | Home directory | P, A | Node P has a read miss at address A; request data and make P a read sharer. |
| Write miss | Local cache | Home directory | P, A | Node P has a write miss at address A; request data and make P the exclusive owner. |
| Invalidate | Local cache | Home directory | A | Request to send invalidates to all remote caches that are caching the block at address A. |
| Invalidate | Home directory | Remote cache | A | Invalidate a shared copy of data at address A. |
| Fetch | Home directory | Remote cache | A | Fetch the block at address A and send it to its home directory; change the state of A in the remote cache to shared. |
| Fetch/invalidate | Home directory | Remote cache | A | Fetch the block at address A and send it to its home directory; invalidate the block in the cache. |
| Data value reply | Home directory | Local cache | D | Return a data value from the home memory. |
| Data write-back | Remote cache | Home directory | A, D | Write-back a data value for address A. |

## 5.5 Directory Actions by Block State

| Block state | Event | Directory action | New state |
| --- | --- | --- | --- |
| Uncached | Read miss | Requesting node is sent the requested data and is made the only sharing node | Shared |
| Uncached | Write miss | Requesting node is sent the requested data and becomes the sharing node | Exclusive |
| Shared | Read miss | Requesting node is sent the data from memory; node is added to sharing set | Shared |
| Shared | Write miss | Requesting node is sent the value; all nodes in the sharing set are sent invalidate messages; sharing set only contains the requesting node | Exclusive |
| Exclusive | Read miss | Owner is sent a data fetch message; owner sends data to the directory; data written back to memory; sharers set contains old owner and requestor | Shared |
| Exclusive | Data write-back | Block becomes uncached, sharer set is empty | Uncached |
| Exclusive | Write miss | Message sent to old owner to invalidate and send the value to the directory; requestor becomes new owner | Exclusive |

### Directory state diagram (slide 35) in short

- Uncached → Shared: read miss; data value reply; Sharers = {P}.

- Uncached → Modify: write miss; data value reply; Sharers = {P}.

- Shared → Shared: read miss; data value reply; Sharers = Sharers + {P}.

- Shared → Modify: write miss; invalidate; Sharers = {P}; data value reply.

- Modify → Shared: read miss; fetch; data value reply; Sharers = Sharers + {P}.

- Modify → Modify: write miss; fetch/invalidate; data value reply; Sharers = {P}.

- Modify → Uncached: data write-back; Sharers = {}.

| Snooping | Directory |
| --- | --- |
| Broadcast on a bus; every cache watches | Point-to-point messages to home directory / specific nodes |
| Sharing state held in each cache | Sharing state held in the directory (presence bits + dirty bit) |
| Good for small machines; bus bandwidth bottleneck | Scales better; suits distributed memory (NUMA) |

SECTION SUMMARY Directory = per-block record of state (Uncached/Shared/Modified) and sharers. Home node owns the memory; owner node holds the modified copy. Misses go to the home directory, which sends fetch / invalidate messages point-to-point.

# PART 6 – Synchronization

## 6.1 Why and Types

- To ensure consistency of shared data structures.

| Type of synchronization | Mechanism |
| --- | --- |
| Mutual exclusion | lock–unlock pairs |
| Point-to-point event synchronization | flags |
| Global event synchronization | barriers |

- Hardware – speed. Software – cost, flexibility, and adaptability.

- A lock ensures that only one process enters the critical section. Hardware locks: lock lines on the bus; lock locations in memory; lock registers.

## 6.2 Simple Software Lock (and its flaw)

lock:   ld  register, location   /* copy location to register */

cmp register, #0         /* compare with 0 */

bnz lock                 /* if not 0, try again */

st  location, #1         /* store 1 to mark it locked */

ret                      /* return control to caller */

unlock: st  location, #0         /* write 0 to location */

ret                      /* return control to caller */

PROBLEM Two processes can enter the critical section – the load/compare and the store are separate, so another process can sneak in between them. Hence atomic operations are needed.

## 6.3 Atomic Read-Modify-Write Primitives

- Hardware primitives to implement synchronization: atomic test-and-set, swap, fetch-and-increment, etc.

- Basic building blocks require memory read and write in an uninterruptable instruction.

| Primitive | Operation |
| --- | --- |
| Atomic exchange / Swap | Atomically swaps the values in a memory location and a register |
| Test-and-set | Value in a memory location is read into a specified register; constant 1 is stored into the location atomically; successful if the value loaded into the register is 0. (Other constants could be used instead of 0 and 1.) “Sets under condition.” |
| Fetch & op (fetch&increment, fetch&decrement, fetch&add) | Atomically reads the current value of a location into a register and writes the new value into the location. Fetch-and-increment: reads original value from memory and increments it in memory. |
| Compare & swap | Three operands: a memory location, register to compare with, register to swap with. Not commonly supported by RISC instruction sets. |

lock:   t&s  register, location

bnz  register, lock      /* if not 0, try again */

ret                      /* return control to caller */

unlock: st   location, #0        /* write 0 to location */

ret                      /* return control to caller */

### Examples in real architectures

| Architecture | Support |
| --- | --- |
| IBM 370 | Included atomic compare&swap for multiprogramming |
| x86 | Any instruction can be prefixed with a lock modifier |
| SPARC | Atomic register-memory ops (swap, compare&swap) |
| MIPS, IBM Power | No atomic operations, but a pair of instructions (load-locked, store-conditional) |

## 6.4 Load-Linked / Store-Conditional (LL/SC)

- Implementing a single atomic memory operation is complex – so use a pair of instructions to implement synchronization.

- LL reads a memory location (synchronization variable) into a register.

- Followed by arbitrary instructions to modify the register value.

- SC tries to write the data of a register to the memory location – if and only if no other write to the location since the processor’s LL; indicated by condition codes or a return value.

- If SC succeeds, all three steps happened atomically. If it fails, it does not write or generate invalidations.

- [In other words: if the contents of the location specified by the LL are changed before the SC to the same address, the store conditional fails.]

lock:   ll   r1, location

bnz  r1, lock            ; location is non-zero => already locked, retry

sc   location, r2        ; try to store (r2 holds the locked value, 1)

beqz lock                ; SC failed => retry

ret

unlock: st   location, #0

ret

### How LL/SC is implemented in hardware

- Requires a lock flag and a lock address register at each processor.

- An LL operation sets the lock flag and puts the address of the read block in the lock address register.

- Incoming invalidation (or update) requests are matched against the lock address register; a successful match resets the lock flag.

- SC checks the lock flag to know whether an intervening write has occurred.

- The lock flag can also be reset when: the lock variable is replaced from the cache; a context switch happens between LL and SC.

- [Slides 46–48 illustrate this: P1 executes LL r1, XYZ (flag = 1, address = XYZ); P2 writes XYZ, causing Invalidate XYZ, which resets P1’s flag to 0; P1’s later SC XYZ, r2 is unsuccessful.]

## 6.5 Implementing Locks – Spin Locks

DEFINITION Spin lock – a lock where the processor repeatedly tests (spins on) the lock variable until it becomes available.

### If no coherence (no cache)

DADDUI R2,R0,#1

lockit: EXCH   R2,0(R1)        ;atomic exchange

BNEZ   R2,lockit       ;already locked?

### If coherence (spin on the cached copy)

lockit: LD     R2,0(R1)        ;load of lock

BNEZ   R2,lockit       ;not available-spin

DADDUI R2,R0,#1        ;load locked value

EXCH   R2,0(R1)        ;swap

BNEZ   R2,lockit       ;branch if lock wasn’t 0

KEY POINT Advantage of this scheme: reduces memory traffic. Processors spin on their cached copy (read hits, no bus traffic) and only attempt the atomic swap when the lock appears free.

### Spin-lock example with coherence (three processors P0, P1, P2)

| Step | P0 | P1 | P2 | Coherence state of lock at end of step | Bus/directory activity |
| --- | --- | --- | --- | --- | --- |
| 1 | Has lock | Begins spin, testing if lock = 0 | Begins spin, testing if lock = 0 | Shared | Cache misses for P1 and P2 satisfied in either order. Lock state becomes shared. |
| 2 | Set lock to 0 | (Invalidate received) | (Invalidate received) | Exclusive (P0) | Write invalidate of lock variable from P0. |
| 3 |  | Cache miss | Cache miss | Shared | Bus/directory services P2 cache miss; write-back from P0; state shared. |
| 4 |  | (Waits while bus/directory busy) | Lock = 0 test succeeds | Shared | Cache miss for P2 satisfied. |
| 5 |  | Lock = 0 | Executes swap, gets cache miss | Shared | Cache miss for P1 satisfied. |
| 6 |  | Executes swap, gets cache miss | Completes swap: returns 0 and sets lock = 1 | Exclusive (P2) | Bus/directory services P2 cache miss; generates invalidate; lock is exclusive. |
| 7 |  | Swap completes and returns 1, and sets lock = 1 | Enter critical section | Exclusive (P1) | Bus/directory services P1 cache miss; sends invalidate and generates write-back from P2. |
| 8 |  | Spins, testing if lock = 0 |  |  | None |

SECTION SUMMARY Synchronization = lock/unlock, flags, barriers. A plain load-compare-store lock is broken; use atomic exchange, test-and-set, fetch-and-increment, or LL/SC (MIPS, Power). Spinning on a cached copy before the swap reduces bus traffic.

# PART 7 – Models of Memory Consistency

## 7.1 The Problem

Processor 1:        Processor 2:

A=0                 B=0

...                 ...

A=1                 B=1

if (B==0) ...       if (A==0) ...

- It should be impossible for both if-statements to be evaluated as true.

- Possible cause: delayed write invalidate?

- Consistency: specification of the allowed behaviour of multithreaded programs executing with shared memory.

- Implications for both programmer and system designer:

  - Programmer uses it to reason about possible results and correctness.

  - System designer can use it to constrain how much accesses can be reordered by compiler or hardware.

## 7.2 Sequential Consistency (SC)

DEFINITION “A multiprocessor is sequentially consistent if the result of any execution is the same as if the operations of all the processors were executed in some sequential order, and the operations of each individual processor appear in this sequence in the order specified by its program.” – Lamport

- Result of execution should be the same as long as: accesses on each processor were kept in order; accesses on different processors were arbitrarily interleaved.

- To implement: delay completion of all memory accesses until all invalidations caused by the access are completed – reduces performance!

### Example: x = y = 0 initially

| Core C1 | Core C2 |
| --- | --- |
| S1: x = NEW | S2: y = NEW |
| L1: r1 = y | L2: r2 = x |

| Outcome (r1, r2) | Allowed under SC? | Reason |
| --- | --- | --- |
| (0, NEW) | ✓ Yes | C1 runs S1, L1 before C2 runs S2, L2 |
| (NEW, 0) | ✓ Yes | C2 runs S2, L2 before C1 runs S1, L1 |
| (NEW, NEW) | ✓ Yes | Both stores happen before both loads |
| (0, 0) | ✗ No (forbidden by SC) | Would need L1 before S2 and L2 before S1, while S1 before L1 and S2 before L2 – a cycle |

### Alternatives to enforcing SC in hardware

- Program-enforced synchronization to force the write on one processor to occur before the read on the other processor.

  - Requires a synchronization object for A and another for B: “Unlock” after write; “Lock” after read.

## 7.3 Write Buffers and (0, 0)

- Processors use write buffers to hold committed stores until the stores are written back.

- Write buffers are not a problem for a single-core processor.

- Multi-core processors can produce the result (r1, r2) = (0, 0), which is forbidden by SC.

## 7.4 Total Store Order (TSO)

- TSO requires that each core preserves the program order of its loads and stores:

| Order | SC | TSO |
| --- | --- | --- |
| Load → Load | Kept | Kept |
| Load → Store | Kept | Kept |
| Store → Store | Kept | Kept |
| Store → Load | Kept | Omitted (relaxed) |

- All cores insert their loads and stores into the global order (<m) respecting their program order (<p):

If L(a) <p L(b)  ⇒  L(a) <m L(b)

If L(a) <p S(b)  ⇒  L(a) <m S(b)

If S(a) <p S(b)  ⇒  S(a) <m S(b)        /* Enable FIFO write buffer */

- Every load gets its value from the last store before it to the same address:

Value of L(a) = Value of Max<m { S(a) | S(a) <m L(a) or S(a) <p L(a) }        /* Need bypassing */

EXAMPLE TSO example: C1: S1: x = NEW; L1: r1 = x; L2: r2 = y. C2: S2: y = NEW; L3: r3 = y; L4: r4 = x. (Initially x = y = 0; assume r2 = r4 = 0.) Each core’s load of its own store is satisfied by bypass from the write buffer: (r1, r3) = (NEW, NEW). The loads of the other core’s variable still see 0: (r2, r4) = (0, 0). Allowed in TSO, not in SC.

### Synchronization example (flag and data)

| Core C1 | Core C2 | Comments |
| --- | --- | --- |
| S1: data1 = NEW S2: data2 = NEW S3: flag = SET | L1: r1 = flag B1: if (r1 != SET) goto L1 L2: r2 = data1 L3: r3 = data2 | Initially data = 0, flag != SET. L1 & B1 may repeat many times. |

- Programmers’ expected orders: S1 → S3 → L1 loads SET → L2;  S2 → S3 → L1 loads SET → L3.

- In addition, SC and TSO also require S1 → S2 and L2 → L3 – not needed by the program for correct operation.

## 7.5 Relaxed Consistency Model (XC)

- XC assumes that a global order exists.

- XC provides a FENCE instruction – FENCE neither specifies an address nor affects the order of memory operations at other cores.

- XC’s memory order is guaranteed to respect program order for: Load → FENCE; Store → FENCE; FENCE → FENCE; FENCE → Load; FENCE → Store.

- XC maintains TSO rules for ordering two accesses to the same address only – loads can immediately see updates due to their own stores.

| Core C1 | Core C2 | Comments |
| --- | --- | --- |
| S1: data1 = NEW S2: data2 = NEW F1: FENCE S3: flag = SET | L1: r1 = flag B1: if (r1 != SET) goto L1 F2: FENCE L2: r2 = data1 L3: r3 = data2 | Initially data = 0, flag != SET. L1 & B1 may repeat many times. |

- FENCEs ensure that: S1, S2 → F1 → S3 → L1 loads SET → F2 → L2, L3.

## 7.6 Relaxed Consistency Models – Rules

- X → Y means: operation X must complete before operation Y is done.

- Sequential consistency requires: R → W, R → R, W → R, W → W.

| Relaxation | Name of model |
| --- | --- |
| Relax W → R | “Total store ordering” |
| Relax W → W | “Partial store order” |
| Relax R → W and R → R | “Weak ordering” and “release consistency” |

- Consistency model is multiprocessor specific.

- Programmers will often implement explicit synchronization.

- Speculation gives much of the performance advantage of relaxed models with sequential consistency. Basic idea: if an invalidation arrives for a result that has not been committed, use speculation recovery.

MNEMONIC Order of strictness: SC (strongest) → TSO (relax W→R) → PSO (also W→W) → Weak ordering / Release consistency (also R→R, R→W). More relaxed = faster hardware, more work for the programmer (fences/locks).

SECTION SUMMARY SC forbids (0,0); write buffers can produce it. TSO relaxes Store→Load; XC relaxes everything except around FENCEs and same-address accesses. Programmers use explicit synchronization; speculation recovers much of the lost performance.

# QUICK REVISION – Comparison Tables & Key Terms

## Key Terms

| Term | One-line meaning |
| --- | --- |
| TLP | Thread-level parallelism: multiple PCs, MIMD, tightly-coupled shared memory |
| Grain size | Amount of computation assigned to each thread |
| SMP / UMA | Few cores, single memory, uniform latency |
| DSM / NUMA | Memory distributed among processors, non-uniform latency |
| Coherence | Reads return most recent write; writes to a location serialized |
| Consistency | When a written value is returned by a read; ordering across locations |
| Migration / Replication | Movement of data / multiple copies of data |
| Write invalidate | On write, invalidate all other copies |
| Write update | On write, update all copies |
| MSI / MESI / MOESI | Invalid-Shared-Modified; + Exclusive (clean, single copy); + Owned |
| True / False sharing miss | Same word shared / different words in same block |
| Home / Owner node | Node holding the memory / node holding the valid modified copy |
| LL / SC | Load-linked / store-conditional pair for atomic synchronization |
| Spin lock | Lock acquired by repeatedly testing the lock variable |
| FENCE | Instruction ordering memory operations around it (XC) |

## Consistency Models at a Glance

| Model | Relaxed order(s) | Needs |
| --- | --- | --- |
| Sequential consistency (SC) | None | All of R→W, R→R, W→R, W→W kept |
| Total store order (TSO) | W → R | Write buffer with bypassing |
| Partial store order | W → R, W → W | Explicit sync for write ordering |
| Weak ordering / release consistency | Also R → R, R → W | Synchronization operations / FENCE |
| XC | Everything except around FENCEs and same-address accesses | FENCE instructions |

## Likely Exam Questions (from slide coverage)

- Explain the cache coherence problem and what coherence means. Differentiate coherence and consistency.

- Compare snooping and directory-based protocols. Explain write invalidate vs write update with an example.

- Explain the MSI snooping protocol with its state-transition diagram. What are MESI and the Owned state?

- Explain true sharing and false sharing misses.

- Explain directory-based coherence: states, messages and actions for read/write miss.

- Explain atomic exchange, test-and-set, fetch-and-increment and LL/SC; implement a spin lock.

- Explain sequential consistency and total store order with the (r1, r2) = (0, 0) example. What are relaxed consistency models?